# Mathematical Model for BLDC motor

Electrical dependent equations:

Ud=ra+LasIs+KeΩs

Where,

Ud is DC bus voltage

ra is winding restiance

La is winding equivalent line inductance

Ka is back emf equivalent line inductance

Ω is mechanical angular speed of motor

I is winding phase current in steady state

Tes=KTIs

Where,

Teis Electromagnetic torque

KTis motor torque coefficient

Mechancial dependent equation:

Tes-T1s=Js+ByΩs

T1 is load torque

J is moment of inertia of rotor

By is coefficient of viscous friction

Electrical transfer function

IsUds-KeΩs=1ra+Las

Mechanical transfer function

ΩsTes-T1s=1Js+By

# Reference:

[1] https://en.wikipedia.org/wiki/Brushless_DC_electric_motor

[2] https://nptel.ac.in/courses/108/104/108104140/

[3] https://nptel.ac.in/courses/108/104/108104140/

[4] https://in.mathworks.com/solutions/power-electronics-control/bldc-motor-control.html

[5] https://in.mathworks.com/help/physmod/sps/ug/brushless-dc motor.html;jsessionid=283cdd2f251e9415f534737e8b66

[6] https://ieeexplore.ieee.org/document/8081083

[7] http://webfiles.portal.chalmers.se/et/MSc/BaldurssonStefanMSc.pdf

[8] http://web.mit.edu/first/scooter/motormath.pdf